# SAFETY DATA SHEET

Alcohol Gel

SECTION 1: Identification: Product identifier and chemical identity

## Product identifier

Product name	Alcohol Gel

## Internal identification	ALCGEL

Relevant identified uses of the substance or mixture and uses advised against Application	Hand Sanitiser

Uses advised against	Use only for intended applications.

## Details of the supplier of the safety data sheet

Supplier	Sampson Chemical Products 42 Redcliffe Gardens Drive Clontarf, QLD, 4019

+61 7 3283 4511

sampson_office@bigpond.com

Contact Person	Poisons Information 131126 or Brennan Stark 0428 835 855 Manufacturer	Eco Pro Australia Pty Ltd

42 Redcliffe Gardens Drive, Clontarf, QLD, 4019

+61 7 3283 4511

sampson_office@bigpond.com

## Emergency telephone number

Emergency telephone	Poisons Information 131126 or Brennan Stark 0428 835 855

## National emergency telephone 131 126 number

SECTION 2: Hazard(s) identification

## Classification of the substance or mixture

Physical hazards	Flam. Liq. 3 - H226

Health hazards	Eye Irrit. 2A - H319 Environmental hazards	Not Classified

## Label elements Hazard pictograms

Signal word	WARNING

Hazard statements	H226 Flammable liquid and vapour.

H319 Causes serious eye irritation.

# Alcohol Gel

Precautionary statements	P210 Keep away from heat/ sparks/ open flames/ hot surfaces. - No smoking.

P233 Keep container tightly closed.

P241 Use explosion-proof electrical equipment. P242 Use only non-sparking tools.

P243 Take precautionary measures against static discharge.

P280 Wear protective gloves/ protective clothing/ eye protection/ face protection. P370+P378 In case of fire: Use foam, carbon dioxide, dry powder or water fog for extinction. P403+P235 Store in a well-ventilated place. Keep cool.

P501 Dispose of contents/ container in accordance with national regulations.

## Other hazards

This product does not contain any substances classified as PBT (persistent, bioaccumulative and toxic) or vPvB (very persistent and very bioaccumulative).

SECTION 3: Composition and information on ingredients

## Mixtures

Ethanol

CAS number: 64-17-5

70%

SECTION 4: First aid measures

## Description of first aid measures

General information	If in doubt, get medical attention promptly. Show this Safety Data Sheet to the medical personnel.

Inhalation	Move affected person to fresh air and keep warm and at rest in a position comfortable for breathing. Loosen tight clothing such as collar, tie or belt. Get medical attention if symptoms are severe or persist.

Ingestion	Rinse mouth thoroughly with water. Get medical advice/attention if you feel unwell. Do not induce vomiting unless under the direction of medical personnel.

Skin Contact	Rinse with water.

Eye contact	Rinse with water. Get medical attention if any discomfort continues.

Protection of first aiders	First aid personnel should wear appropriate protective equipment during any rescue. Most important symptoms and effects, both acute and delayed

General information	The severity of the symptoms described will vary dependent on the concentration and the length of exposure.

Inhalation	No specific symptoms known.

Ingestion	No specific symptoms known.

Skin contact	No specific symptoms known.

Eye contact	No specific symptoms known. May be slightly irritating to eyes. Indication of any immediate medical attention and special treatment needed

Notes for the doctor	Treat symptomatically.

SECTION 5: Firefighting measures

## Extinguishing media

Suitable extinguishing media	The product is flammable. Extinguish with alcohol-resistant foam, carbon dioxide, dry powder or water fog. Use fire-extinguishing media suitable for the surrounding fire.

# Alcohol Gel

## Unsuitable extinguishing media

Do not use water jet as an extinguisher, as this will spread the fire.

## Special hazards arising from the substance or mixture

Specific hazards	Containers can burst violently or explode when heated, due to excessive pressure build-up. Flammable liquid and vapour. Vapours may be ignited by a spark, a hot surface or an ember. Vapours may form explosive mixtures with air. Fire-water run-off in sewers may create fire or explosion hazard.

Hazardous combustion products

Advice for firefighters

Protective actions during firefighting

Special protective equipment for firefighters

Thermal decomposition or combustion products may include the following substances: Harmful gases or vapours.

Avoid breathing fire gases or vapours. Evacuate area. Cool containers exposed to heat with water spray and remove them from the fire area if it can be done without risk. Cool containers exposed to flames with water until well after the fire is out. If a leak or spill has not ignited, use water spray to disperse vapours and protect men stopping the leak. Control run-off water by containing and keeping it out of sewers and watercourses. If risk of water pollution occurs, notify appropriate authorities.

Wear positive-pressure self-contained breathing apparatus (SCBA) and appropriate protective clothing. Firefighter's clothing conforming to Australia/New Zealand Standards AS/NZS 4967 (for clothing) AS/NZS 1801 (for helmets), AS/NZS 4821 (for protective boots), AS/NZS 1801 (for protective gloves) will provide a basic level of protection for chemical incidents.

## Hazchem Code	•2Y

SECTION 6: Accidental release measures

## Personal precautions, protective equipment and emergency procedures

Personal precautions	Wear protective clothing as described in Section 8 of this safety data sheet. No action shall be taken without appropriate training or involving any personal risk. Evacuate area. Provide adequate ventilation. No smoking, sparks, flames or other sources of ignition near spillage.

Promptly remove any clothing that becomes contaminated.

## Environmental precautions

Environmental precautions	Avoid discharge into drains or watercourses or onto the ground.

## Methods and material for containment and cleaning up

Methods for cleaning up	Wear protective clothing as described in Section 8 of this safety data sheet. Clear up spills immediately and dispose of waste safely. Eliminate all ignition sources if safe to do so. No smoking, sparks, flames or other sources of ignition near spillage. Do not allow material to enter confined spaces, due to the risk of explosion. Absorb small quantities with paper towels and evaporate in a safe place. Once evaporation is complete, place paper in a suitable waste disposal container and seal securely. Flush contaminated area with plenty of water. Wash thoroughly after dealing with a spillage. For waste disposal, see Section 13.

## Reference to other sections

Reference to other sections	For personal protection, see Section 8. See Section 11 for additional information on health hazards. See Section 12 for additional information on ecological hazards. For waste disposal, see Section 13.

SECTION 7: Handling and storage, including how the chemical may be safely used

Precautions for safe handling

# Alcohol Gel

Usage precautions	Read and follow manufacturer's recommendations. Handle all packages and containers carefully to minimise spills. Keep container tightly sealed when not in use. Avoid the formation of mists. The product is flammable. Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. Do not handle until all safety precautions have been read and understood.

## Advice on general occupational hygiene

Avoid eye contact and repeated or prolonged skin contact. Do not inhale any vapour or mist.

## Conditions for safe storage, including any incompatibilities

Storage precautions	Keep away from oxidising materials, heat and flames. Keep only in the original container.

Keep container tightly closed, in a cool, well ventilated place. Keep containers upright. Protect containers from damage.

Storage class	Flammable liquid storage.

## Specific end use(s)

Specific end use(s)	The identified uses for this product are detailed in Section 1.

SECTION 8: Exposure controls and personal protection

## Control parameters Occupational exposure limits Ethanol

Long-term exposure limit (8-hour TWA): 1000 ppm 1880 mg/m³ 2-methylpropan-2-ol

Long-term exposure limit (8-hour TWA): 100 ppm 303 mg/m³ Short-term exposure limit (15-minute): 150 ppm 455 mg/m³

## Exposure controls Protective equipment

Appropriate engineering controls

Provide adequate ventilation. Observe any occupational exposure limits for the product or ingredients.

Eye/face protection	Unless the assessment indicates a higher degree of protection is required, the following protection should be worn: Tight-fitting safety glasses.

Hand protection	No specific hand protection recommended.

## Other skin and body protection

Wear appropriate clothing to prevent repeated or prolonged skin contact.

Hygiene measures	Wash hands thoroughly after handling. Wash at the end of each work shift and before eating, smoking and using the toilet. Do not eat, drink or smoke when using this product.

Respiratory protection	Ensure all respiratory protective equipment is suitable for its intended use and complies with Australia/New Zealand Standard AS/NZS 1716. Check that the respirator fits tightly and the filter is changed regularly. Gas and combination filter cartridges should comply with Australia/New Zealand Standard AS/NZS 1716. Full face mask respirators with replaceable filter cartridges should comply with Australia/New Zealand Standard AS/NZS 1716. Half mask and quarter mask respirators with replaceable filter cartridges should comply with Australia/New Zealand Standard AS/NZS 1716.

# Alcohol Gel

## Environmental exposure controls

Keep container tightly sealed when not in use.

SECTION 9: Physical and chemical properties

## Information on basic physical and chemical properties Appearance	Viscous liquid.

Colour	Clear.

pH	6.0 - 7.0

## Specific Gravity

Flash point	~ 25°C Abel

SECTION 10: Stability and reactivity

Reactivity	There are no known reactivity hazards associated with this product.

Stability	Stable at normal ambient temperatures and when used as recommended. Stable under the prescribed storage conditions.

## Possibility of hazardous reactions

The following materials may react strongly with the product: Oxidising agents.

Conditions to avoid	Avoid heat, flames and other sources of ignition. Containers can burst violently or explode when heated, due to excessive pressure build-up. Static electricity and formation of sparks must be prevented.

Materials to avoid	Oxidising materials. Oxidising agents.

## Hazardous decomposition products

Does not decompose when used and stored as recommended. Thermal decomposition or combustion products may include the following substances: Harmful gases or vapours.

SECTION 11: Toxicological information

## Information on toxicological effects Acute toxicity - oral

Notes (oral LD₅₀)	Based on available data the classification criteria are not met.

## Acute toxicity - dermal

Notes (dermal LD₅₀)	Based on available data the classification criteria are not met.

## Acute toxicity - inhalation

Notes (inhalation LC₅₀)	Based on available data the classification criteria are not met.

## Skin corrosion/irritation

Animal data	Based on available data the classification criteria are not met.

## Serious eye damage/irritation

Serious eye damage/irritation  Based on available data the classification criteria are not met.

## Respiratory sensitisation

Respiratory sensitisation	Based on available data the classification criteria are not met.

## Skin sensitisation

Skin sensitisation	Based on available data the classification criteria are not met.

# Alcohol Gel

## Germ cell mutagenicity

Genotoxicity - in vitro	Based on available data the classification criteria are not met.

## Carcinogenicity

Carcinogenicity	Based on available data the classification criteria are not met.

IARC carcinogenicity	Contains a substance/a group of substances which may cause cancer. IARC Group 1 Carcinogenic to humans.

## Reproductive toxicity

Reproductive toxicity - fertility  Based on available data the classification criteria are not met.

## Reproductive toxicity - development

Based on available data the classification criteria are not met.

## Specific target organ toxicity - single exposure

STOT - single exposure	Not classified as a specific target organ toxicant after a single exposure.

## Specific target organ toxicity - repeated exposure

STOT - repeated exposure	Not classified as a specific target organ toxicant after repeated exposure.

## Aspiration hazard

Aspiration hazard	Based on available data the classification criteria are not met.

General information	The severity of the symptoms described will vary dependent on the concentration and the length of exposure.

Inhalation	No specific symptoms known.

Ingestion	No specific symptoms known.

Skin Contact	No specific symptoms known.

Eye contact	No specific symptoms known.

SECTION 12: Ecological information

Route of exposure	Ingestion Inhalation Skin and/or eye contact Target Organs	No specific target organs known.

Ecotoxicity	Not regarded as dangerous for the environment. However, large or frequent spills may have hazardous effects on the environment.

Toxicity	Based on available data the classification criteria are not met. Persistence and degradability

Persistence and degradability The degradability of the product is not known.

## Bioaccumulative potential

Bioaccumulative Potential	No data available on bioaccumulation.

## Mobility in soil

Mobility	No data available.

## Other adverse effects

Other adverse effects	None known.

SECTION 13: Disposal considerations

# Alcohol Gel

## Waste treatment methods

General information	The generation of waste should be minimised or avoided wherever possible. Reuse or recycle products wherever possible. This material and its container must be disposed of in a safe way. When handling waste, the safety precautions applying to handling of the product should be considered. Care should be taken when handling emptied containers that have not been thoroughly cleaned or rinsed out. Empty containers or liners may retain some product residues and hence be potentially hazardous.

Disposal methods	Dispose of surplus products and those that cannot be recycled via a licensed waste disposal contractor. Waste, residues, empty containers, discarded work clothes and contaminated cleaning materials should be collected in designated containers, labelled with their contents. Incineration or landfill should only be considered when recycling is not feasible.

SECTION 14: Transport information

## UN number

UN No. (ADG)	1170

UN No. (IMDG)	1170

UN No. (ICAO)	1170

UN proper shipping name

Proper shipping name (ADG) ETHANOL (ETHYL ALCOHOL) or ETHANOL SOLUTION (ETHYL ALCOHOL SOLUTION)

## Proper shipping name (IMDG)

ETHANOL (ETHYL ALCOHOL) or ETHANOL SOLUTION (ETHYL ALCOHOL SOLUTION)

Proper shipping name (ICAO) ETHANOL (ETHYL ALCOHOL) or ETHANOL SOLUTION (ETHYL ALCOHOL SOLUTION)

Transport hazard class(es)

ADG class	3

ADG classification code	F1

ADG label	3

IMDG class	3

ICAO class/division	3

Transport labels

Packing group

ADG packing group	III

IMDG packing group	III

ICAO packing group	III

Environmental hazards

Environmentally hazardous substance/marine pollutant No.

Special precautions for user

EmS	F-E, S-D

# Alcohol Gel

## Hazchem Code	•2Y

Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code

Not applicable.

SECTION 15: Regulatory information

## Inventories Australia - AICS

None of the ingredients are listed or exempt.

SECTION 16: Any other relevant information

Training advice	Only trained personnel should use this material.

Revision date	19/01/2026

## Revision	5

Supersedes date	6/05/2021

SDS No.	4603

General Information	The following risk and hazard statements are to be considered a glossary. They relate to the raw materials used in this product and therefore may not be accurate for the finished product itself. For the complete risk and hazard statements for this product please refer to section 2 of this Safety Data Sheet

Hazard statements in full	H226 Flammable liquid and vapour.

H319 Causes serious eye irritation.

This information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process. Such information is, to the best of the company's knowledge and belief, accurate and reliable as of the date indicated. However, no warranty, guarantee or representation is made to its accuracy, reliability or completeness. It is the user's responsibility to satisfy himself as to the suitability of such information for his own particular use.