# SAFETY DATA SHEET

Jubilee Oven and Grill

SECTION 1: Identification: Product identifier and chemical identity

## Product identifier

Product name	Jubilee Oven and Grill

Internal identification	JUBOVEN

Relevant identified uses of the substance or mixture and uses advised against Application	High alkaline cleaner for ovens, grills and baking surfaces Uses advised against	Use only for intended applications.

## Details of the supplier of the safety data sheet

Supplier	Sampson Chemical Products 42 Redcliffe Gardens Drive Clontarf, QLD, 4019

+61 7 3283 4511

sampson_office@bigpond.com

Contact Person	Poisons Information 131126 or Brennan Stark 0428 835 855 Manufacturer	Eco Pro Australia Pty Ltd

42 Redcliffe Gardens Drive, Clontarf, QLD, 4019

+61 7 3283 4511

sampson_office@bigpond.com

## Emergency telephone number

Emergency telephone	Poisons Information 131126 or Brennan Stark 0428 835 855

SECTION 2: Hazard(s) identification

## Classification of the substance or mixture Physical hazards	Not Classified

Health hazards	Skin Corr. 1A - H314 Eye Dam. 1 - H318 Environmental hazards	Not Classified

## Label elements Hazard pictograms

Signal word	DANGER

Hazard statements	H314 Causes severe skin burns and eye damage.

# Jubilee Oven and Grill

Precautionary statements	P260 Do not breathe vapour/ spray.

P264 Wash contaminated skin thoroughly after handling.

P280 Wear protective gloves/ protective clothing/ eye protection/ face protection. P301+P330+P331 IF SWALLOWED: Rinse mouth. Do NOT induce vomiting.

P303+P361+P353 IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water/ shower.

P304+P340 IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P310 Immediately call a POISON CENTER or doctor/ physician. P321 Specific treatment (see medical advice on this label).

P363 Wash contaminated clothing before reuse. P405 Store locked up.

P501 Dispose of contents/ container in accordance with national regulations.

Contains	Caustic Soda, Caustic Potash, Alkylpolyglycoside C8-10 Other hazards

This product does not contain any substances classified as PBT (persistent, bioaccumulative and toxic) or vPvB (very persistent and very bioaccumulative).

| SECTION 3: Composition and information on ingredients |  |
| --- | --- |
| Mixtures |  |
| Caustic Soda CAS number: 1310-73-2 | 10-30% |
| Caustic Potash CAS number: 1310-58-3 | 1-10% |
| Alkylpolyglycoside C8-10 CAS number: 68515-73-1 | 1-10% |
| EDTA NA4 CAS number: 10378-23-1 | 1-10% |
| SECTION 4: First aid measures |  |
| Description of first aid measures |  |

General information	Get medical attention if any discomfort continues. Show this Safety Data Sheet to the medical personnel. Chemical burns must be treated by a physician.

Inhalation	Move affected person to fresh air and keep warm and at rest in a position comfortable for breathing. Maintain an open airway. Loosen tight clothing such as collar, tie or belt. Rinse nose and mouth with water. Never give anything by mouth to an unconscious person. Get medical attention if symptoms are severe or persist.

Ingestion	Rinse mouth thoroughly with water. Give a few small glasses of water or milk to drink. Stop if the affected person feels sick as vomiting may be dangerous. Get medical attention.

Skin Contact	It is important to remove the substance from the skin immediately. Rinse immediately with plenty of water. Continue to rinse for at least 15 minutes and get medical attention. Chemical burns must be treated by a physician.

# Jubilee Oven and Grill

Eye contact	Rinse immediately with plenty of water. Do not rub eye. Remove any contact lenses and open eyelids wide apart. Continue to rinse for at least 15 minutes and get medical attention.

Protection of first aiders	It may be dangerous for first aid personnel to carry out mouth-to-mouth resuscitation.

## Most important symptoms and effects, both acute and delayed

General information	The severity of the symptoms described will vary dependent on the concentration and the length of exposure.

Inhalation	A single exposure may cause the following adverse effects: Corrosive to the respiratory tract.

Symptoms following overexposure may include the following: Severe irritation of nose and throat.

Ingestion	May cause chemical burns in mouth, oesophagus and stomach. Symptoms following overexposure may include the following: Severe stomach pain. Nausea, vomiting.

Skin contact	Causes severe burns. Symptoms following overexposure may include the following: Pain or irritation. Redness. Blistering may occur.

Eye contact	Causes serious eye damage. Symptoms following overexposure may include the following: Pain. Profuse watering of the eyes. Redness.

## Indication of any immediate medical attention and special treatment needed Notes for the doctor	Treat symptomatically.

SECTION 5: Firefighting measures

## Extinguishing media

Suitable extinguishing media	The product is not flammable. Extinguish with alcohol-resistant foam, carbon dioxide, dry powder or water fog. Use fire-extinguishing media suitable for the surrounding fire.

## Unsuitable extinguishing media

Do not use water jet as an extinguisher, as this will spread the fire.

## Special hazards arising from the substance or mixture

Specific hazards	Containers can burst violently or explode when heated, due to excessive pressure build-up.

Severe corrosive hazard. Water used for fire extinguishing, which has been in contact with the product, may be corrosive.

Hazardous combustion products

Advice for firefighters

Protective actions during firefighting

Special protective equipment for firefighters

Thermal decomposition or combustion products may include the following substances: Very toxic or corrosive gases or vapours.

Avoid breathing fire gases or vapours. Evacuate area. Keep upwind to avoid inhalation of gases, vapours, fumes and smoke. Cool containers exposed to heat with water spray and remove them from the fire area if it can be done without risk. Cool containers exposed to flames with water until well after the fire is out. If a leak or spill has not ignited, use water spray to disperse vapours and protect men stopping the leak. Avoid discharge to the aquatic environment. Control run-off water by containing and keeping it out of sewers and watercourses. If risk of water pollution occurs, notify appropriate authorities.

Regular protection may not be safe. Wear chemical protective suit. Wear positive-pressure self-contained breathing apparatus (SCBA) and appropriate protective clothing. Firefighter's clothing conforming to Australia/New Zealand Standards AS/NZS 4967 (for clothing) AS/NZS 1801 (for helmets), AS/NZS 4821 (for protective boots), AS/NZS 1801 (for protective gloves) will provide a basic level of protection for chemical incidents.

Hazchem Code	2X

# Jubilee Oven and Grill

SECTION 6: Accidental release measures

## Personal precautions, protective equipment and emergency procedures

Personal precautions	Wear protective clothing as described in Section 8 of this safety data sheet. No action shall be taken without appropriate training or involving any personal risk. Avoid inhalation of dust and vapours. Use suitable respiratory protection if ventilation is inadequate. Avoid contact with skin and eyes.

## Environmental precautions

Environmental precautions	The product may affect the acidity (pH) of water which may have hazardous effects on aquatic organisms. Avoid discharge to the aquatic environment.

## Methods and material for containment and cleaning up

Methods for cleaning up	Wear protective clothing as described in Section 8 of this safety data sheet. Clear up spills immediately and dispose of waste safely. This product is corrosive. Small Spillages: Collect spillage. Large Spillages: Absorb spillage with non-combustible, absorbent material. The contaminated absorbent may pose the same hazard as the spilled material. Collect and place in suitable waste disposal containers and seal securely. Label the containers containing waste and contaminated materials and remove from the area as soon as possible. Flush contaminated area with plenty of water. Wash thoroughly after dealing with a spillage. For waste disposal, see Section 13.

## Reference to other sections

Reference to other sections	For personal protection, see Section 8. See Section 11 for additional information on health hazards. See Section 12 for additional information on ecological hazards. For waste disposal, see Section 13.

SECTION 7: Handling and storage, including how the chemical may be safely used

## Precautions for safe handling

Usage precautions	Read and follow manufacturer's recommendations. Wear protective clothing as described in Section 8 of this safety data sheet. Keep away from food, drink and animal feeding stuffs.

Handle all packages and containers carefully to minimise spills. Keep container tightly sealed when not in use. Avoid the formation of mists. This product is corrosive. Immediate first aid is imperative. Do not handle until all safety precautions have been read and understood. Do not handle broken packages without protective equipment. Do not reuse empty containers.

## Advice on general occupational hygiene

Wash promptly if skin becomes contaminated. Take off contaminated clothing and wash before reuse. Wash contaminated clothing before reuse.

## Conditions for safe storage, including any incompatibilities

Storage precautions	Store locked up. Keep only in the original container. Keep container tightly closed, in a cool, well ventilated place. Keep containers upright. Protect containers from damage. Store away from incompatible materials (see Section 10).

Storage class	Corrosive storage.

## Specific end use(s)

Specific end use(s)	The identified uses for this product are detailed in Section 1.

SECTION 8: Exposure controls and personal protection

## Control parameters Occupational exposure limits Caustic Soda

Ceiling value: 2 mg/m³

# Jubilee Oven and Grill

Caustic Potash Ceiling value: 2 mg/m³

## Exposure controls Protective equipment

Appropriate engineering controls

Provide adequate ventilation. Observe any occupational exposure limits for the product or ingredients.

Eye/face protection	Wear tight-fitting, chemical splash goggles or face shield. If inhalation hazards exist, a full-face respirator may be required instead.

Hand protection	Wear protective gloves. The most suitable glove should be chosen in consultation with the glove supplier/manufacturer, who can provide information about the breakthrough time of the glove material. To protect hands from chemicals, gloves should comply with Australia/New Zealand Standard AS/NZS 2161. Considering the data specified by the glove manufacturer, check during use that the gloves are retaining their protective properties and change them as soon as any deterioration is detected. Frequent changes are recommended.

## Other skin and body protection

Wear appropriate clothing to prevent any possibility of skin contact.

Hygiene measures	Wash hands thoroughly after handling. Wash at the end of each work shift and before eating, smoking and using the toilet. Do not eat, drink or smoke when using this product.

Respiratory protection	Ensure all respiratory protective equipment is suitable for its intended use and complies with Australia/New Zealand Standard AS/NZS 1716. Check that the respirator fits tightly and the filter is changed regularly. Gas and combination filter cartridges should comply with Australia/New Zealand Standard AS/NZS 1716. Full face mask respirators with replaceable filter cartridges should comply with Australia/New Zealand Standard AS/NZS 1716. Half mask and quarter mask respirators with replaceable filter cartridges should comply with Australia/New Zealand Standard AS/NZS 1716.

## Environmental exposure controls

Keep container tightly sealed when not in use. Emissions from ventilation or work process equipment should be checked to ensure they comply with the requirements of environmental protection legislation. In some cases, fume scrubbers, filters or engineering modifications to the process equipment will be necessary to reduce emissions to acceptable levels.

SECTION 9: Physical and chemical properties

## Information on basic physical and chemical properties Appearance	Coloured liquid.

Colour	Dark brown.

Odour	Typical chemical odour

pH	14

Specific Gravity	1.12

SECTION 10: Stability and reactivity

Solubility(ies)	Soluble in water.

Reactivity	There are no known reactivity hazards associated with this product.

# Jubilee Oven and Grill

Stability	Stable at normal ambient temperatures and when used as recommended. Stable under the prescribed storage conditions.

## Possibility of hazardous reactions

No potentially hazardous reactions known.

Conditions to avoid	Avoid heat, flames and other sources of ignition.

Materials to avoid	Reacts violently with acids in an exothermic reaction. Reacts with oxidising agents. May be corrosive to metals.

## Hazardous decomposition products

Does not decompose when used and stored as recommended. Thermal decomposition or combustion products may include the following substances: Corrosive gases or vapours.

SECTION 11: Toxicological information

## Information on toxicological effects Acute toxicity - oral

Notes (oral LD₅₀)	Based on available data the classification criteria are not met.

## ATE oral (mg/kg)	24,600.0 Acute toxicity - dermal

Notes (dermal LD₅₀)	Based on available data the classification criteria are not met.

## Acute toxicity - inhalation

Notes (inhalation LC₅₀)	Based on available data the classification criteria are not met.

## Skin corrosion/irritation

Animal data	Skin Corr. 1A - H314 Causes severe burns.

## Serious eye damage/irritation

Serious eye damage/irritation  Eye Dam. 1 - H318 Corrosive to skin. Corrosivity to eyes is assumed.

## Respiratory sensitisation

Respiratory sensitisation	Based on available data the classification criteria are not met.

## Skin sensitisation

Skin sensitisation	Based on available data the classification criteria are not met.

## Germ cell mutagenicity

Genotoxicity - in vitro	Based on available data the classification criteria are not met.

## Carcinogenicity

Carcinogenicity	Based on available data the classification criteria are not met. IARC carcinogenicity	None of the ingredients are listed or exempt.

## Reproductive toxicity

Reproductive toxicity - fertility  Based on available data the classification criteria are not met.

## Reproductive toxicity - development

Based on available data the classification criteria are not met.

## Specific target organ toxicity - single exposure

STOT - single exposure	Not classified as a specific target organ toxicant after a single exposure.

## Specific target organ toxicity - repeated exposure

STOT - repeated exposure	Not classified as a specific target organ toxicant after repeated exposure.

# Jubilee Oven and Grill

## Aspiration hazard

Aspiration hazard	Based on available data the classification criteria are not met.

General information	The severity of the symptoms described will vary dependent on the concentration and the length of exposure.

Inhalation	Corrosive to the respiratory tract. Symptoms following overexposure may include the following: Severe irritation of nose and throat.

Ingestion	May cause chemical burns in mouth, oesophagus and stomach. Symptoms following overexposure may include the following: Severe stomach pain. Nausea, vomiting.

Skin Contact	Causes severe burns. Symptoms following overexposure may include the following: Pain or irritation. Redness. Blistering may occur.

Eye contact	Causes serious eye damage. Symptoms following overexposure may include the following: Pain. Profuse watering of the eyes. Redness.

SECTION 12: Ecological information

Route of exposure	Ingestion Inhalation Skin and/or eye contact Target Organs	No specific target organs known.

Ecotoxicity	Not regarded as dangerous for the environment. However, large or frequent spills may have hazardous effects on the environment.

Toxicity	Based on available data the classification criteria are not met. Persistence and degradability

Persistence and degradability The degradability of the product is not known.

## Bioaccumulative potential

Bioaccumulative Potential	No data available on bioaccumulation.

## Mobility in soil

Mobility	No data available.

## Other adverse effects

Other adverse effects	None known.

SECTION 13: Disposal considerations

## Waste treatment methods

General information	The generation of waste should be minimised or avoided wherever possible. Reuse or recycle products wherever possible. This material and its container must be disposed of in a safe way. When handling waste, the safety precautions applying to handling of the product should be considered. Care should be taken when handling emptied containers that have not been thoroughly cleaned or rinsed out. Empty containers or liners may retain some product residues and hence be potentially hazardous.

Disposal methods	Dispose of surplus products and those that cannot be recycled via a licensed waste disposal contractor. Waste, residues, empty containers, discarded work clothes and contaminated cleaning materials should be collected in designated containers, labelled with their contents. Incineration or landfill should only be considered when recycling is not feasible.

SECTION 14: Transport information

# Jubilee Oven and Grill

## UN number

UN No. (ADG)	1760

UN No. (IMDG)	1760

UN No. (ICAO)	1760

UN proper shipping name

Proper shipping name (ADG) CORROSIVE LIQUID, N.O.S. (CONTAINS Caustic Soda, Caustic Potash)

## Proper shipping name (IMDG)

CORROSIVE LIQUID, N.O.S. (CONTAINS Caustic Soda, Caustic Potash)

Proper shipping name (ICAO) CORROSIVE LIQUID, N.O.S. (CONTAINS Caustic Soda, Caustic Potash)

Transport hazard class(es)

ADG class	8

ADG classification code	C9

ADG label	8

IMDG class	8

ICAO class/division	8

Transport labels

Packing group

ADG packing group	II

IMDG packing group	II

ICAO packing group	II

Environmental hazards

Environmentally hazardous substance/marine pollutant No.

Special precautions for user

EmS	F-A, S-B

## Hazchem Code	2X

Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code

Not applicable.

SECTION 15: Regulatory information

## Inventories Australia - AICS

None of the ingredients are listed or exempt.

SECTION 16: Any other relevant information

# Jubilee Oven and Grill

Training advice	Only trained personnel should use this material.

Revision date	29/01/2026

## Revision	4

Supersedes date	27/07/2021

SDS No.	4800

General Information	The following risk and hazard statements are to be considered a glossary. They relate to the raw materials used in this product and therefore may not be accurate for the finished product itself. For the complete risk and hazard statements for this product please refer to section 2 of this Safety Data Sheet

Hazard statements in full	H314 Causes severe skin burns and eye damage.

H318 Causes serious eye damage.

This information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process. Such information is, to the best of the company's knowledge and belief, accurate and reliable as of the date indicated. However, no warranty, guarantee or representation is made to its accuracy, reliability or completeness. It is the user's responsibility to satisfy himself as to the suitability of such information for his own particular use.